OPEN SOURCE HARDWARE CERTIFICATION

Application & Documentation Package

JALSHUDDHI

Open Source River-Water Purification & Monitoring System

Modular inline unit: sensor chamber · multi-stage filtration · UV disinfection · ESP32 monitoring head

| Creator | Sayaan Mehta |
| --- | --- |
| Mentor | Satyam Prakash |
| Country of origin | India |
| Project version | 1.0.0 |
| Submission target | OSHWA Certification — application.oshwa.org/apply |

This document is the complete, submission-ready certification package. Italicised notes mark the few fields you must replace (contact email, repository URL) before applying.

# Contents

# 1. Certification Application Summary

OSHWA certification is a free, self-certification programme run by the Open Source Hardware Association. A creator confirms that their hardware meets the community Open Source Hardware Definition, submits the online form, and on approval receives a unique identifier (UID) and the right to use the certification mark. The table below contains every value the online application asks for, ready to transcribe.

| Responsible party | Sayaan Mehta |
| --- | --- |
| Public contact | [add project / student email] Required by the form — replace before submitting. |
| Country of origin | India Sets the UID prefix (Indian projects use the IN prefix). |
| Project name | JALSHUDDHI |
| Project website | [add public repository URL, e.g. github.com/<user>/jalshuddhi] Must be live and contain the files in Section 7 before you apply. |
| Project version | 1.0.0 |
| Primary type | Environmental |
| Additional type | Science (Education is an equally valid choice) |
| Hardware licence | CERN-OHL-S v2.0 |
| Software licence | MIT |
| Documentation licence | CC BY-SA 4.0 |
| Documentation URL | [same public repository URL] |
| OSHWA UID | Assigned on approval (format INxxxxxx) You do not choose this; OSHWA issues it. |

Project description (paste into the form's description field):

JALSHUDDHI is a low-cost, repairable, inline water purifier with built-in water-quality monitoring. It draws raw river or canal water, passes it through a multi-stage filtration train (screen, sediment, carbon and ultrafiltration) followed by UV disinfection, and uses an ESP32 to measure pH, TDS, turbidity and temperature. Treated water is released to the clean outlet only when all four parameters fall inside a safe drinking-water window; otherwise the unit diverts to waste and displays a warning. It is designed for households that depend on surface water where no piped supply exists.

# 2. Project Overview

JALSHUDDHI (from the Hindi/Sanskrit for “water purification”) is an open source hardware project that pairs a conventional multi-stage water-purification train with a small electronic monitoring system. The combination matters: many low-cost purifiers clean water but give the user no way to know whether the output is actually safe. JALSHUDDHI measures the treated water in real time and physically withholds it from the clean outlet until the readings are within a safe range.

The entire creator contribution — the enclosure design, the system integration, the wiring, and the firmware — is released under open licences so that any maker, school, or community group can reproduce, repair, and improve the unit. Off-the-shelf modules (the ESP32 board, sensor breakouts, pumps, UV lamp and filter cartridges) are standard components with publicly available datasheets.

### Project name note

“JALSHUDDHI” is a descriptive working name. OSHWA notes that descriptive names are harder to protect as trademarks; if the maker wants a stronger brand later, a more distinctive coined name can be adopted without affecting certification.

# 3. Problem Statement & Motivation

The project takes Karnal district in Haryana, India as its reference setting. The need it addresses is the gap between the number of households and the number with a safe, authorised water supply.

- Municipal records for the Karnal Municipal Corporation area indicate that of about 116,051 assessed properties, only roughly 40,957 hold a legal water (and sewer) connection — meaning on the order of 64.7% of properties lack an authorised piped-water supply into the premises.

- In the western parts of the district, groundwater is frequently saline, which pushes communities toward canal-fed schemes. Where piped infrastructure has not yet been laid, households fall back on surface water — largely the Yamuna and its canals — for everyday use.

- Planned pipeline expansion (for example under AMRUT-2) has faced delays in technical approval, funding and clearances, so the gap persists in newly regularised colonies and peripheral areas.

- The problem is not unique to Karnal: WHO and UNICEF have estimated that roughly one in three people worldwide do not have access to safely managed drinking water.

Against this backdrop, a purifier that is cheap to build, easy to repair from locally available parts, openly documented, and able to tell the user whether the water is safe has clear social value. The maker's motivation also draws on community work carried out through the Vikas Foundation, a youth initiative focused on helping under-served communities.

# 4. System Architecture & Design

## 4.1 Filtration & disinfection train

Water flows through the following stages. Each stage targets a different class of contaminant, ending with a barrier (ultrafiltration) and disinfection (UV).

Submersible Pump → Screen Filter → Pre-Filter → Sediment → Pre-Carbon → Post-Carbon → Solenoid Valve → Booster Pump → UF Membrane → UV Chamber → Clean outlet

| Stage | Purpose |
| --- | --- |
| Submersible pump | Draws raw water from the river/canal source into the unit. |
| Screen filter | Coarse Y-type strainer that removes grit, leaves and large particulate at the intake. |
| Pre-filter | Spun polypropylene filter for finer suspended solids. |
| Sediment cartridge | Removes remaining fine sediment (~5 micron) to protect downstream media. |
| Pre-carbon | Activated carbon stage that adsorbs chlorine, odour and organic compounds. |
| Post-carbon | Polishing carbon stage that improves taste and removes residual organics. |
| Solenoid valve | Electrically controlled diverter — sends first-pass / unsafe water to waste. |
| Booster pump | Raises pressure to push water through the ultrafiltration membrane. |
| UF membrane | Ultrafiltration (~0.01 micron) barrier against bacteria, cysts and colloids. |
| UV chamber | UV-C disinfection of any surviving micro-organisms before the clean outlet. |

## 4.2 Water-quality monitoring subsystem

An ESP32 microcontroller reads four parameters from probes mounted at the treated-water (outlet) side, so the values reflect the water the user is about to receive.

- pH — two-point calibrated analog probe, temperature-compensated.

- TDS — total dissolved solids in ppm, with standard temperature compensation.

- Turbidity — optical sensor reporting NTU, clamped to a clear-water baseline.

- Temperature — DS18B20 digital probe, also used to compensate the pH and TDS readings.

Live values are shown on a 16×2 I²C LCD in the controller head, and streamed over the serial port for debugging. An on-board LED indicates at a glance whether the current water is within the safe window.

## 4.3 Control logic & safety

The firmware runs a non-blocking finite state machine. Crucially, water reaches the clean outlet only in the RUNNING state, and only while every parameter is within range; the moment a reading drifts out of bounds the unit moves to HOLD and diverts to waste.

| State | Behaviour |
| --- | --- |
| IDLE | All actuators off. Waits for the user to press the start button. |
| PRIMING | Intake pump fills the filter train; outlet still closed. |
| FLUSHING | First-pass water is sent to waste through the solenoid valve. |
| RUNNING | Booster pump and UV lamp on, valve closed → water flows to the clean outlet while readings stay safe. |
| HOLD_UNSAFE | A parameter is out of range: booster stops, valve diverts to waste, LCD warns. Resumes automatically when readings recover. |
| FAULT | Temperature probe missing/disconnected: everything off, error shown. |

Default safe window (editable to match the local standard, e.g. BIS 10500): pH 6.5–8.5, TDS ≤ 500 ppm, turbidity ≤ 5 NTU. These are defined as named constants at the top of the firmware.

## 4.4 Physical enclosure

The enclosure is a modular inline body, designed for 3D printing, with four functional sections:

- Sensor mounting chamber — houses the pH, TDS, turbidity and temperature probes.

- Filtration system — the central body holding the cartridge stack.

- Pump section — the booster pump.

- ESP32 + display module — the angled head carrying the controller, LCD and start button.

The original CAD source files (and exported STLs) are the project's primary open design files and must accompany the certification (see Section 7).

# 5. Bill of Materials

The complete machine-readable BOM ships as bom.csv in the repository. “Third-party” items are off-the-shelf modules with public datasheets; the enclosure is the creator's own open design.

| Component | Notes / Spec | Origin |
| --- | --- | --- |
| Y-type screen filter | Coarse pre-screen on intake | Third-party |
| Pre-filter (spun PP) | Push-fit transparent housing | Third-party |
| Sediment cartridge | ~5 micron sediment stage | Third-party |
| Pre-carbon / Post-carbon | Activated carbon stages | Third-party |
| UF membrane | Ultrafiltration ~0.01 micron | Third-party |
| UV lamp kit | UV-C lamp + ballast (shrouded) | Third-party |
| Submersible pump | Intake pump for raw water | Third-party |
| Booster pump | ~100 GPD RO booster pump | Third-party |
| Solenoid valve | 24 V normally-closed diverter | Third-party |
| Pipe + push-fit connectors | 1/4 in tubing + fittings | Third-party |
| Analog pH probe | Gravity-type analog pH board | Third-party |
| Analog TDS probe | Gravity-type analog TDS board | Third-party |
| Turbidity sensor | SEN0189-class analog sensor | Third-party |
| Temperature probe | DS18B20 waterproof + 4.7k pull-up | Third-party |
| ESP32 DevKit | ESP32-WROOM-32, 38-pin | Third-party |
| LCD display | 16x2 character LCD, I2C backpack | Third-party |
| Relay board | 4-channel opto-isolated module | Third-party |
| 24 V adapter + buck converter | Mains→24 V; 24 V→5 V/3.3 V logic | Third-party |
| Enclosure (modular) | 3D-printed; CAD in hardware/cad | Creator (open) |

# 6. Firmware

The firmware is a single, well-commented Arduino sketch for the ESP32 (jalshuddhi_firmware.ino). It is released under the MIT licence. Engineering choices were made deliberately for reliability on low-cost hardware:

- Non-blocking loop — all timing is millis()-based, so the display, sensors and safety checks keep running while pumps are active (no delay() in the main loop).

- Median-filtered sampling — each analog parameter is read as the median of 21 samples to reject the voltage spikes that plague cheap water probes.

- Temperature compensation — pH and TDS are corrected against the live temperature reading.

- Safety-first defaults — on boot and on stop, every actuator is switched off; water only reaches the clean outlet when readings are in range.

- Single CONFIG block — all pins, calibration constants and safe-water limits live at the top of the file, the only values a builder normally edits.

### Pin map

| Signal | GPIO | Notes |
| --- | --- | --- |
| pH analog out | 35 | ADC1, input-only |
| TDS analog out | 34 | ADC1, input-only |
| Turbidity analog out | 32 | ADC1; scale 5V→3.3V if needed |
| DS18B20 data | 33 | 4.7k pull-up to 3V3 |
| I2C SDA / SCL (LCD) | 21 / 22 | 16x2 I2C display |
| Start/Stop button | 18 | to GND, internal pull-up |
| Submersible pump relay | 25 | active-LOW relay IN1 |
| Booster pump relay | 26 | relay IN2 |
| Solenoid valve relay | 27 | relay IN3 |
| UV lamp relay | 14 | relay IN4 |
| Status LED | 2 | on-board; lit = safe to drink |

Build chain: Arduino IDE + the Espressif ESP32 board package; libraries OneWire, DallasTemperature and LiquidCrystal_I2C (all open source, installed from the Library Manager). Calibrate the pH and TDS probes once before field use, as described in the file header.

# 7. Documentation Package (OSHWA Checklist)

OSHWA's documentation requirements come from the Open Source Hardware Definition: certified hardware must publish design files and allow their modification and distribution. The checklist below maps OSHWA's items to what this project provides.

| OSHWA item | Required? | Status |
| --- | --- | --- |
| Original design files publicly available | Yes | Add CAD/STL to hardware/cad |
| Auxiliary design files | Optional | Wiring diagram in wiring.md |
| Bill of materials publicly available | Yes | Provided — bom.csv |
| Photos at stages of assembly | Optional | Add to docs/ before launch |
| Instructions / explanations | Optional | Provided — README + this doc |
| Design files properly open-licensed | Yes | CERN-OHL-S / CC BY-SA / MIT |

### Repository layout (the public URL you submit)

jalshuddhi/
  README.md             documentation hub
  LICENSE               MIT (software)
  firmware/jalshuddhi_firmware.ino
  hardware/device_render.png
  hardware/bom.csv
  hardware/wiring.md
  hardware/cad/         ▸ ADD original CAD + STL
  docs/assembly.md      ▸ ADD photo build guide
  docs/calibration.md   ▸ ADD probe calibration

The three items marked ▸ ADD are the only outstanding pieces. The CAD source is the single most important one — without published, modifiable design files the project cannot be certified.

# 8. Open Source Licensing Plan

OSHWA asks for a separate licence for hardware, software and documentation. None may carry a “NonCommercial” or “NoDerivatives” restriction, because the definition requires that others be free to modify and commercially reuse the work.

| Component | Licence | Why |
| --- | --- | --- |
| Hardware | CERN-OHL-S v2.0 | Purpose-built strongly-reciprocal licence for the enclosure CAD and wiring design; derivatives stay open. CC BY-SA 4.0 is an accepted, simpler alternative. |
| Software | MIT | Permissive and beginner-friendly for the firmware; lets anyone reuse the control code. GPLv3 is an alternative if reciprocal sharing of firmware changes is preferred. |
| Documentation | CC BY-SA 4.0 | OSHWA-recommended; allows copying and building on the docs with attribution and share-alike. |
| Branding | (not opened) | The name “JALSHUDDHI” may be kept as a trademark; branding is outside the scope of opening the source and does not affect certification. |

# 9. Creator Contribution & Third-Party Components

OSHWA's “creator contribution” rule requires that everything under the creator's control be open; it does not require opening third-party components the creator did not design. It does require that those third-party parts have publicly accessible datasheets.

### Open (creator's own work)

- The modular enclosure design (CAD source + exported STLs).

- System integration: the choice and arrangement of filtration stages and the fluid path.

- The wiring design and pin mapping.

- The ESP32 firmware, including the safety state machine and sensor processing.

- All project documentation (this package, the README, diagrams).

### Third-party (off-the-shelf, datasheets public)

- ESP32 module, LCD, relay board, buck converter and 24 V adapter.

- Analog pH, TDS and turbidity sensor boards; DS18B20 temperature probe.

- Pumps, solenoid valve, filter cartridges and the UV lamp kit.

Because the creator's contribution is fully open and the third-party modules are standard parts with accessible documentation, the project satisfies OSHWA's requirement even though not every physical component is itself open hardware.

# 10. Compliance with the Open Source Hardware Definition

| Definition requirement | How JALSHUDDHI meets it |
| --- | --- |
| Documentation & design files released | Enclosure CAD/STL, wiring, BOM and firmware published in the public repository. |
| Modification & distribution permitted | CERN-OHL-S, MIT and CC BY-SA all permit modification and redistribution. |
| Commercial use allowed | None of the chosen licences carry NonCommercial or NoDerivatives clauses. |
| Software provided & openly licensed | Firmware released under MIT; relies only on open source libraries. |
| Third-party parts identified | BOM marks every off-the-shelf module; datasheets are publicly available. |
| Clear, separate licences | Distinct licences declared for hardware, software and documentation. |

# 11. Submission Checklist & Next Steps

To turn this package into a live certification:

- Create a public repository (GitHub/GitLab) and upload everything in Section 7, including the CAD source files and exported STLs.

- Add the licence files: LICENSE (MIT) for software, the CERN-OHL-S notice for hardware, and a CC BY-SA 4.0 notice for the documentation.

- Add a few assembly photos and a short calibration note to docs/ (optional but strengthens copyright protection).

- Go to application.oshwa.org/apply and transcribe the values from Section 1; paste the description; provide the repository URL as both the project website and documentation URL.

- Submit. OSHWA reviews the self-certification and, on approval, issues your UID (INxxxxxx) and the right to display the certification mark for one year (renewable).

Note: OSHWA certification is the recognised programme for designating hardware as open source; it is free and there is no separate monetary prize. Certification is what lets the project carry the official open-source-hardware mark and appear in the global certified-projects directory.

# Appendix A — References

Background and requirements draw on the following sources. Statistics are paraphrased; figures should be re-verified against the primary sources before publication.

- Open Source Hardware Association — Certification requirements, process, documentation and licensing pages (certification.oshwa.org).

- Open Source Hardware Definition (oshwa.org/definition).

- The Tribune — reporting on legal vs. unconnected water/sewer properties in the Karnal Municipal Corporation area.

- Central Ground Water Board — Aquifer Mapping and Management Plan for Karnal District (groundwater salinity and reliance on canal/surface water).

- Press Information Bureau / AMRUT-2 coverage — delays in piped-water rollout in Haryana.

- WHO / UNICEF (2019) — global estimate that one in three people lack safely managed drinking water.

- Project source materials supplied by the creator: “Current project” brief, “List of Materials”, “Understanding the scenario”, “Karnal — Pipe Water Connection” and the design deck.